NetDevOps 第七天作业

[APIFOX, RESTCONF采集CPU利用率并配置SYSLOG]

- APIFOX, RESTCONF采集CPU利用率并配置SYSLOG

任务一:安装APIFOX, 加载下面链接教主提供的项目文件(当然有能力自己做更好):

链接: https://pan.baidu.com/s/1-KrfjyJMZk9rt-JOJ4C9jA 提取码: c5ir

任务二:使用APIFOX获取最近5秒CPU利用率, 测试截图如下! 请你转换到Python3代码,做成函数让代码更具通用性(def monitor_cpu(device_ip, username, password, monitor_type='5s'):)

任务三:使用APIFOX配置SYSLOG(host与trap level), 测试截图如下! 请你转换到Python3代码,做成函数让代码更具通用性(def conf_syslog(device_ip, username, password, severity, hostip):)

作业标准:

- 提供完整的代码（Windows 或 Linux）

- 对打印结果进行截图

作业内容如下

任务一：安装APIFOX

安装APIFOX 并导入了 CPU 和logging的部分API数据

任务二:使用APIFOX获取最近5秒CPU利用率

用APIFOX获取

用python获取

python代码如下

import requests

from requests.auth import HTTPBasicAuth

import urllib3

# 禁用 HTTPS 证书告警（实验环境常用）

urllib3.disable_warnings(urllib3.exceptions.InsecureRequestWarning)

# === RESTCONF 访问配置 ===

url = "https://192.0.2.210:443/restconf/data/Cisco-IOS-XE-process-cpu-oper:cpu-usage/cpu-utilization/five-seconds"

username = "admin"

password = "cisco123"

# === RESTCONF Header ===

headers = {

"Accept": "application/yang-data+json",

"Content-Type": "application/yang-data+json"

}

# === 发送 GET 请求 ===

response = requests.get(

url,

auth=HTTPBasicAuth(username, password),

headers=headers,

verify=False  # 实验设备一般使用自签名证书

)

# === 解析返回 ===

if response.status_code == 200:

print("请求成功")

print(response.json())

else:

print(f"请求失败，状态码: {response.status_code}")

print(response.text)

任务三:使用APIFOX配置SYSLOG

用APIFOX配置SYSLOG

检查设备配置APIFOX请求生效

用python put另一套配置

检查设备配置python请求生效

python代码如下

import requests

from requests.auth import HTTPBasicAuth

import urllib3

import json

# 禁用 HTTPS 证书告警（实验环境常用）

urllib3.disable_warnings(urllib3.exceptions.InsecureRequestWarning)

url = "https://192.0.2.210:443/restconf/data/Cisco-IOS-XE-native:native/logging"

username = "admin"

password = "cisco123"

headers = {

"Accept": "application/yang-data+json",

"Content-Type": "application/yang-data+json"

}

data = {

"Cisco-IOS-XE-native:logging": {

"trap": {"severity": "debugging"},

"host": {

"ipv4-host-list": [

{"ipv4-host": "192.168.100.1"},

{"ipv4-host": "192.168.100.5"},

{"ipv4-host": "192.168.100.3"}

]

},

"hostip": "192.168.100.1"

}

}

resp = requests.put(

url,

headers=headers,

data=json.dumps(data),

auth=HTTPBasicAuth(username, password),

verify=False

)

print(resp.status_code, resp.text)